Architectural and engineering specifications

JK18W2

• The subwoofer loudspeaker system shall have one ferrite magnet 18-inch low frequency transducer loaded in a band-pass tuned enclosure. It shall be rated at a nominal impedance of 8 ohms.

• The usable bandwidth shall be 34Hz to 190Hz (-10dB).

• SPL, measured at 1W and 1m under free field conditions shall be 99dB.

• Maximum continuous musical program shall be 2000W.

• Enclosure shall be band-pass type with some lateral rectangular ducts.

• Enclosure cabinet construction shall consist of birch plywood with a black textured finish. Dimensions shall be 740 mm (29,1 inch) wide, 552 mm (21,7 inch) high, 710 mm (27,9 inch) deep. Enclosure weight shall be 47,8 Kg.

• The front enclosure shall be protected by two steel grilles with acoustically transparent foam. The grilles shall be secured via screws to keep it in place when facing the cabinet downwards.

• The cabinet shall feature a round socket with M20 thread located on the top panel to allow full-range enclosure coupling.

• Electrical connections shall be made via one of the two 4-pole NL4MPR Speakon connectors with the following configuration: +1/-1 signal input, +2/-2 disconnected.

• The loudspeaker system shall be the Master Audio® Joker JK18W2.